ACTUATOR SIZING FOR SORTING MECHANISM

For the sizing of the actuator used to move the gate ,I have decided that it must be a servo motor to be able to move the gate between 2 exact positions , however I had to do some calculations in order to find the model needed to achieve the needed motion.

# 1. Gate Physical Properties

The following data was extracted directly from Autodesk Inventor iProperties for the gate2 component. The gate is manufactured from MDF Medium Density Fiberboard with a density of 0.800 g/cm³.

| Parameter | Value | Unit |
| --- | --- | --- |
| Mass | 0.001 | kg (~1 g) |
| Volume | 1643.915 | mm³ |
| Surface Area | 1428.832 | mm² |
| CoG Offset X | -1.223 | mm |
| CoG Offset Y | -2.168 | mm |
| CoG Offset Z | 1.500 | mm |
| Principal Moment I1 | 0.400 | kg·mm² |
| Principal Moment I2 | 0.009 | kg·mm² |
| Principal Moment I3 | 0.398 | kg·mm² |
| Material | MDF Medium Density Fiberboard | — |
| Density | 0.800 | g/cm³ |

# 2. Operating Parameters

The following operating conditions were specified for the sorting mechanism:

| Parameter | Value | Notes |
| --- | --- | --- |
| Swing Range | 40° – 120° | 80° total travel |
| Operating Speed | < 0.3 s per stroke | Fast sorting cycle |

# 3. Torque Analysis

## 3.1  Effective Moment of Inertia

For a center-pivot gate, the effective rotational inertia about the pivot axis is taken as the average of the two principal moments in the plane of rotation (I1 and I3):

I_eff: (I1 + I3) / 2 = (0.400 + 0.398) / 2 = 0.399 kg·mm² = 3.99 × 10⁻⁷ kg·m²

## 3.2  Static Gravitational Torque

The moment arm is the horizontal projection of the CoG offset from the pivot centre. The dominant offset is in Y (2.168 mm):

τ_grav: m × g × r_CoG = 0.001 × 9.81 × 0.00217 ≈ 0.0000213 N·m ≈ 0.021 mN·m

## 3.3  Dynamic (Acceleration) Torque

Assuming constant angular acceleration to traverse 80° (1.396 rad) in 0.3 s:

α: 2θ / t² = 2 × 1.396 / 0.3² ≈ 31.0 rad/s²

τ_acc: I_eff × α = 3.99×10⁻⁷ × 31.0 ≈ 0.0124 mN·m ≈ 0.012 mN·m

## 3.4  Design Torque with Safety Factor

A safety factor of 3× is applied to account for payload impact forces, air resistance, and mechanical wear. An additional 10–20% is added for friction losses at the pivot bearing:

τ_design: (τ_grav + τ_acc) × 3 × 1.15 ≈ 0.033 × 3.45 ≈ 0.114 → rounded to 0.12 mN·m

| Torque Component | Value | Formula / Basis |
| --- | --- | --- |
| Static gravitational | 0.021 mN·m | m × g × r_CoG |
| Dynamic (acceleration) | 0.012 mN·m | I_eff × α |
| Raw total | 0.033 mN·m | Static + dynamic |
| With safety factor (3×) | 0.099 mN·m | 3× margin applied |
| Design torque (+ friction) | 0.12 mN·m | Rounded up for losses |

# 4. Servo Selection Criteria

The following minimum specifications are required for the servo actuator. Speed is the binding constraint — the gate mass is negligible (~1 g), so torque margin is very large with any standard hobby servo.

| Parameter | Required | Rationale | Priority |
| --- | --- | --- | --- |
| Stall torque | ≥ 0.5 mN·m | 5× margin over design | Low |
| Speed | ≥ 266 °/s | 80° in < 0.3 s | CRITICAL |
| Rotation range | ≥ 80° travel | Full gate sweep | High |
| Resolution | ≤ 1° preferred | Accurate gate stop | Medium |
| Supply voltage | 3.3 V or 5 V | MCU compatibility | Medium |

# 5. Recommended Servo Options

All three options below provide 150–200× more torque than required. Selection is driven entirely by speed capability and physical size constraints.

| Model | Torque | Speed | Weight | Notes |
| --- | --- | --- | --- | --- |
| MG90S | 22 mN·m | ~500 °/s | 13 g | Metal gear, durable |
| DS-929MG | 30 mN·m | ~460 °/s | 11 g | Higher torque margin |

# 7. Summary

The gate2 component is extremely lightweight (~1 g, MDF) with a symmetrical center-pivot configuration. Torque demand is negligible at 0.12 mN·m design torque. Any standard micro-servo exceeds this by 150× or more. The sole engineering constraint is cycle speed: the mechanism requires 80° of travel in under 0.3 s, demanding a servo rated at ≥ 266 °/s. The SG90/SG92R family (600 °/s) is the optimal choice — compact, fast, inexpensive, and widely available.